# Railway Signaling Control System

Specification Document — Railway Domain

Version 1.0 | Classification: Internal

# 1. Scope

This specification defines the requirements for an electronic interlocking system for railway signaling, covering signal control, point management, track section monitoring, and route-setting logic. The system shall comply with CENELEC EN 50129 (SIL 4).

# 2. Functional Requirements

## 2.1 Signal Control

REQ-001: The system shall control three-aspect color light signals with states: Red (stop), Yellow (caution), and Green (proceed).

REQ-002: The system shall ensure that a signal can only display Green when the entire route ahead is clear and locked.

REQ-003: The system shall automatically set any signal to Red if the track section immediately ahead becomes occupied.

## 2.2 Point/Switch Control

REQ-004: The system shall control railway points (switches) with detection of Normal and Reverse positions.

REQ-005: The system shall lock points in position once a route is set and shall prevent movement until the route is released.

REQ-006: Point movement shall complete within 6 seconds. If detection is not confirmed within 8 seconds, the system shall declare a point failure.

## 2.3 Route Setting

REQ-007: The system shall support automatic route-setting from an origin signal to a destination signal.

REQ-008: The system must verify that all track sections in a route are unoccupied before setting the route.

# 3. Performance Requirements

REQ-009: The system shall process all interlocking logic within a cycle time of 500 ms maximum.

REQ-010: The system shall detect track occupation changes within 200 ms of an axle counter event.

# 4. Safety Requirements

REQ-011: The system shall implement a fail-safe design: any detected failure shall result in signals defaulting to the most restrictive aspect (Red).

REQ-012: The system shall prevent conflicting routes from being set simultaneously (flank protection).

REQ-013: The system shall maintain a safety integrity level of SIL 4 as per EN 50129, with a tolerable hazard rate of 10^-9 per hour.

REQ-014: All safety-critical state transitions must be logged with timestamps for audit purposes, stored for a minimum of 10 years.

# 5. Interface Requirements

REQ-015: The system shall interface with the traffic management system via a standardized RaSTA (Rail Safe Transport Application) protocol.

REQ-016: The system shall provide a local control panel interface for manual override operations in emergency situations.